头环四鱼眼相机标定采集流程_内置SD卡版_日常快速启动版本终版

头环四鱼眼相机标定采集流程 内置 SD 卡版

快速启动版本 3

适用范围：四路相机 camera 标定。流程分为“新头环首次初始化”和“Linux 日常采集”两部分。

黄色内容是换头环或新录制时才需要修改的值。示例头环 IP 为 192.168.10.109，必须以扫描结果为准。

# 固定信息

| 项目 | 固定值或规则 |
| --- | --- |
| 工作站有线 IP | 192.168.10.10 |
| 路由器 LAN | 192.168.10.1 |
| BAG 保存目录 | /media/orangepi/sdcard/data/kalibr_bags |
| ROS Master | 头环本机 http://localhost:11311 |
| 固定 Topic 顺序 | LEFT, RIGHT, BRIGHT, BLEFT |

# 一 新头环从 0 到 1 仅第一次需要显示器

当新头环扫描到 IP，但 Linux 无法 SSH 或 22 端口拒绝连接时，接一次显示器和键盘。完成本节、重启并在 Linux 验收通过后，以后不再连接显示器。

操作窗口：头环本地终端。看到 orangepi@orangepicm5-tablet:~$ 后执行。

Bash echo "=== 1 生成并校验 SSH 密钥 ===" sudo ssh-keygen -A sudo /usr/sbin/sshd -t echo echo "=== 2 启用 SSH ===" sudo systemctl reset-failed ssh sudo systemctl enable ssh sudo systemctl restart ssh echo -n "SSH："; systemctl is-enabled ssh; systemctl is-active ssh sudo ss -ltnp ¦ grep ':22' ¦¦ echo "SSH 22 端口未监听" echo echo "=== 3 启用四相机服务 ===" sudo systemctl enable seeker_cam_bridge.service sudo systemctl restart seeker_cam_bridge.service echo -n "相机服务："; systemctl is-enabled seeker_cam_bridge.service; systemctl is-active seeker_cam_bridge.service echo echo "=== 4 检查内置 SD 卡并创建 BAG 目录 ===" if findmnt -no SOURCE,TARGET,FSTYPE /media/orangepi/sdcard; then mkdir -p /media/orangepi/sdcard/data/kalibr_bags printf "Kalibr BAG directory ready\n" \ > /media/orangepi/sdcard/data/kalibr_bags/write_test.txt sync test -w /media/orangepi/sdcard/data/kalibr_bags \ && echo "BAG 目录：可写 PASS" \ ¦¦ echo "BAG 目录：不可写 FAIL" else echo "SD 卡未自动挂载：不能录制 BAG。" fi echo echo "=== 初始化检查结束 ===".

说明：通过标准：SSH 为 enabled、active；相机服务为 enabled、active；BAG 目录显示“可写 PASS”。命令报错时先不要重启。

操作窗口：头环本地终端。通过后执行重启。

Plain Text sudo reboot.

说明：重启后拔掉显示器。下面开始全部在 Linux 工作站操作。

# 二 Linux 日常采集流程

# 1 发现当前头环 IP

操作窗口：Linux 工作站终端。

Plain Text sudo arp-scan --interface=eno1 192.168.10.0/24.

说明：记录 Realtek 设备的 IP。下文以 192.168.10.109 为例；若扫描结果不同，只替换黄色 IP。

# 2 等待 SSH 就绪

操作窗口：Linux 工作站终端。

Plain Text for i in {1..18}; do date '+%H:%M:%S' nc -vz -w 3 192.168.10.109 22 && echo "SSH 22: PASS，可以远程操作" && break ¦¦ echo "SSH 22: 等待头环开机" sleep 10 done.

说明：若扫描到 IP 但持续显示 Connection refused，说明该头环尚未完成“新头环首次初始化”。

如果首次 SSH 出现 REMOTE HOST IDENTIFICATION HAS CHANGED，并且已确认旧头环不在网络中，执行：

Plain Text ssh-keygen -f "/home/key/.ssh/known_hosts" -R "192.168.10.11".

说明：再重新执行 SSH。新头环复用相同 IP 时，这是正常的主机密钥保护提示。

# 3 检查四路相机

操作窗口：Linux 工作站终端。

Plain Text ssh orangepi@192.168.10.109 ' source /opt/ros/noetic/setup.bash export ROS_MASTER_URI=http://localhost:11311 systemctl is-active seeker_cam_bridge.service for TOPIC in /fisheye/left/image_raw/compressed /fisheye/right/image_raw/compressed /fisheye/bright/image_raw/compressed /fisheye/bleft/image_raw/compressed do timeout 8 rostopic echo -n 1 "$TOPIC" >/dev/null && echo "PASS $TOPIC" ¦¦ echo "FAIL $TOPIC" done '.

说明：必须显示 active 和四个 PASS。出现 FAIL，停止录制。

## 3.1 SSH 提示“主机身份已改变”时处理

操作窗口：Linux 工作站终端 key@key。

**出现条件：**更换了新头环，但路由器又给它分配了旧 IP；SSH 出现 REMOTE HOST IDENTIFICATION HAS CHANGED。
 **前提：**确认旧头环已经断开，只剩当前这台新头环接入网络。

Plain Text ssh-keygen -f "/home/key/.ssh/known_hosts" -R "192.168.10.109"

**注释：**删除 Linux 中保存的“旧头环 .109 的 SSH 身份记录”。
 **每次需要修改：**仅把最后的 192.168.10.109 改成 arp-scan 扫描到的当前头环 IP。

然后重新连接头环：

Plain Text ssh orangepi@192.168.10.109

**注释：**首次连接会询问是否信任新头环的身份密钥；确认当前只有这台头环在线后，输入 yes，再输入头环密码。成功进入后输入 exit 回到 Linux，即可继续执行四路相机检查。

# 4 启动四路预览

操作窗口：Linux 工作站终端 1。该窗口只用于预览。

Plain Text cd /home/key/Downloads/headband_calibration_preview_relay_v1.10.4 ./run_temporary_four_preview.sh 192.168.10.109 192.168.10.10 balanced.

说明：浏览器打开 http://127.0.0.1:8766。采集结束后，在此预览终端按 Ctrl+C。预览只辅助观察，不代表 Kalibr 通过。

# 5 录制正式 BAG

操作窗口：Linux 工作站终端 2。先进入头环。

Plain Text ssh -tt orangepi@192.168.10.109.

操作窗口：头环终端。看到 orangepi@orangepicm5-tablet:~$ 后执行。

Plain Text BAG_NAME="camera_cs0001_1010_1600.bag" source /opt/ros/noetic/setup.bash export ROS_MASTER_URI=http://localhost:11311 cd /media/orangepi/sdcard/data/kalibr_bags echo "开始录制：$(pwd)/$BAG_NAME" rosbag record -O "$BAG_NAME" /fisheye/left/image_raw/compressed /fisheye/right/image_raw/compressed /fisheye/bright/image_raw/compressed /fisheye/bleft/image_raw/compressed.

说明：只改黄色 BAG_NAME。完成采集后，在本窗口按 Ctrl+C；必须等待重新出现 orangepi@ 提示符，不能先断开 SSH。

# 6 检查 BAG 并返回 Linux

操作窗口：头环终端。录制正常结束后执行。

Plain Text rosbag info /media/orangepi/sdcard/data/kalibr_bags/"$BAG_NAME" sync exit.

说明：rosbag info 应显示时长、四路 Topic 和相近消息数。exit 后提示符必须回到 key@key。

# 7 复制 BAG 到 Linux

操作窗口：Linux 工作站终端。禁止在 orangepi@ 提示符下执行。

Plain Text BAG_NAME="camera_cs0001_1010_1600.bag" mkdir -p /home/key/文档/kalibr/data/cs-0001 rsync -avP --partial orangepi@192.168.10.109:/media/orangepi/sdcard/data/kalibr_bags/"$BAG_NAME" /home/key/文档/kalibr/data/live_bags/.

# 8 校验复制结果

操作窗口：Linux 工作站终端。

Plain Text BAG_NAME="camera_cs0001_1010_1600.bag" ssh orangepi@192.168.10.109 "sha256sum /media/orangepi/sdcard/data/kalibr_bags/$BAG_NAME" sha256sum /home/key/文档/kalibr/data/live_bags/$BAG_NAME docker exec kalibr bash -lc "source /opt/ros/noetic/setup.bash && rosbag info /data/live_bags/$BAG_NAME".

说明：两行 SHA256 必须完全相同，且容器内 rosbag info 能读取。

# 9 运行 Kalibr 相机标定    #    每次二选一运行即可，推荐_DS_none版

操作窗口：Linux 工作站终端。

### #运行 Kalibr 相机标定（DS_none版）

Bash ( set -Eeuo pipefail trap 'rc=$?; trap - EXIT; printf "\n脚本结束，退出码：%s\n" "$rc"; read -r -p "按回车关闭窗口..." _ ¦¦ true; exit "$rc"' EXIT # ===== 每次测试新 BAG，只修改以下 3 行 ===== DEVICE_DIR="cp-0001" # 设备目录名，例如 cp-0002、cp-00016、cs-0001 SOURCE_BAG="/home/key/文档/kalibr/data/cp-0001/camera/camera_cp0001_1011_2015.bag" # 本次要测的第一步相机 BAG 完整路径 BAG_FREQ_HZ="10.0" # 抽帧频率；例如 10.0、5.0、2.0 # =========================================== DATA_ROOT="/home/key/文档/kalibr/data" TEST_ROOT="$DATA_ROOT/model_eval_ds_camera_20261012" BAG_NAME="$(basename "$SOURCE_BAG")" SAMPLE_ID="${BAG_NAME%.bag}" RUN_TAG="ds-none_${BAG_FREQ_HZ}hz_$(date -u +%Y%m%dT%H%M%SZ)" RUN_DIR="$TEST_ROOT/$DEVICE_DIR/$SAMPLE_ID/$RUN_TAG" RUN_DIR_INNER="/data/model_eval_ds_camera_20261012/$DEVICE_DIR/$SAMPLE_ID/$RUN_TAG" BASE_NAME="${BAG_NAME%.bag}" CAMCHAIN="$RUN_DIR/${BASE_NAME}-camchain.yaml" RESULTS_TXT="$RUN_DIR/${BASE_NAME}-results-cam.txt" REPORT_PDF="$RUN_DIR/${BASE_NAME}-report-cam.pdf" LOG_FILE="$RUN_DIR/run.log" METADATA="$RUN_DIR/run_metadata.txt" # 计时从粘贴执行命令后开始，包含复制、标定和结果检查 TOTAL_START_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" TOTAL_START_SECONDS="$(date +%s)" if [[ ! -s "$SOURCE_BAG" ]]; then echo "找不到 BAG 或文件为空：$SOURCE_BAG" >&2 exit 1 fi if [[ "$(docker inspect -f '{{.State.Running}}' kalibr 2>/dev/null)" != "true" ]]; then echo "Kalibr 容器 kalibr 未运行，请先启动容器。" >&2 exit 1 fi if [[ -e "$RUN_DIR" ]]; then echo "测试目录已存在，为避免覆盖而停止：$RUN_DIR" >&2 exit 2 fi mkdir -p "$RUN_DIR" cp -v -- "$SOURCE_BAG" "$RUN_DIR/$BAG_NAME" SOURCE_SHA256="$(sha256sum "$SOURCE_BAG" ¦ awk '{print $1}')" COPY_SHA256="$(sha256sum "$RUN_DIR/$BAG_NAME" ¦ awk '{print $1}')" if [[ "$SOURCE_SHA256" != "$COPY_SHA256" ]]; then echo "BAG 副本校验失败，SHA256 不一致，停止标定。" >&2 exit 3 fi echo echo "第一步：DS 四路相机标定" echo "设备：$DEVICE_DIR" echo "BAG：$BAG_NAME" echo "抽帧频率：$BAG_FREQ_HZ Hz" echo "测试副本：$RUN_DIR/$BAG_NAME" echo "SHA256：$COPY_SHA256" echo "隔离输出目录：$RUN_DIR" echo "运行期间请勿关闭终端或按 Ctrl+C。" KALIBR_START_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" KALIBR_START_SECONDS="$(date +%s)" set +e docker exec \ -e BAG_NAME="$BAG_NAME" \ -e BAG_FREQ_HZ="$BAG_FREQ_HZ" \ -e RUN_DIR_INNER="$RUN_DIR_INNER" \ kalibr bash -lc ' set -e source /opt/ros/noetic/setup.bash source /catkin_ws/devel/setup.bash unset KALIBR_MANUAL_FOCAL_LENGTH_INIT export MPLBACKEND=Agg unset DISPLAY cd "$RUN_DIR_INNER" rosrun kalibr kalibr_calibrate_cameras \ --target /data/april_6x6.yaml \ --models ds-none ds-none ds-none ds-none \ --topics \ /fisheye/left/image_raw/compressed \ /fisheye/right/image_raw/compressed \ /fisheye/bright/image_raw/compressed \ /fisheye/bleft/image_raw/compressed \ --bag "$RUN_DIR_INNER/$BAG_NAME" \ --bag-freq "$BAG_FREQ_HZ" \ --dont-show-report ' 2>&1 ¦ tee "$LOG_FILE" KALIBR_EXIT_CODE="${PIPESTATUS[0]}" set -e KALIBR_END_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" KALIBR_END_SECONDS="$(date +%s)" KALIBR_SECONDS="$((KALIBR_END_SECONDS - KALIBR_START_SECONDS))" CAMCHAIN_OK=no RESULTS_OK=no REPORT_OK=no [[ -s "$CAMCHAIN" ]] && CAMCHAIN_OK=yes [[ -s "$RESULTS_TXT" ]] && RESULTS_OK=yes [[ -s "$REPORT_PDF" ]] && REPORT_OK=yes if [[ "$KALIBR_EXIT_CODE" -eq 0 && "$CAMCHAIN_OK" == yes && "$RESULTS_OK" == yes && "$REPORT_OK" == yes ]]; then STATUS=SUCCESS_COMPLETE else STATUS=INCOMPLETE_OR_FAILED fi TOTAL_END_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" TOTAL_END_SECONDS="$(date +%s)" TOTAL_SECONDS="$((TOTAL_END_SECONDS - TOTAL_START_SECONDS))" { printf 'step=step1_camera_calibration\n' printf 'device_dir=%s\n' "$DEVICE_DIR" printf 'sample_id=%s\n' "$SAMPLE_ID" printf 'model=ds-none x4\n' printf 'bag_frequency_hz=%s\n' "$BAG_FREQ_HZ" printf 'source_bag=%s\n' "$SOURCE_BAG" printf 'test_copy=%s/%s\n' "$RUN_DIR" "$BAG_NAME" printf 'source_sha256=%s\n' "$SOURCE_SHA256" printf 'copy_sha256=%s\n' "$COPY_SHA256" printf 'total_start_utc=%s\n' "$TOTAL_START_UTC" printf 'total_end_utc=%s\n' "$TOTAL_END_UTC" printf 'total_wall_seconds=%s\n' "$TOTAL_SECONDS" printf 'total_wall_minutes_seconds=%s分%s秒\n' \ "$((TOTAL_SECONDS / 60))" "$((TOTAL_SECONDS % 60))" printf 'kalibr_start_utc=%s\n' "$KALIBR_START_UTC" printf 'kalibr_end_utc=%s\n' "$KALIBR_END_UTC" printf 'kalibr_wall_seconds=%s\n' "$KALIBR_SECONDS" printf 'kalibr_wall_minutes_seconds=%s分%s秒\n' \ "$((KALIBR_SECONDS / 60))" "$((KALIBR_SECONDS % 60))" printf 'kalibr_exit_code=%s\n' "$KALIBR_EXIT_CODE" printf 'camchain_exists=%s\n' "$CAMCHAIN_OK" printf 'results_exists=%s\n' "$RESULTS_OK" printf 'report_exists=%s\n' "$REPORT_OK" printf 'status=%s\n' "$STATUS" } ¦ tee "$METADATA" echo echo "=== 本次测试文件 ===" ls -lh "$RUN_DIR" echo echo "状态：$STATUS" echo "测试目录：$RUN_DIR" echo "日志：$LOG_FILE" [[ "$STATUS" == SUCCESS_COMPLETE ]] )

### #运行 Kalibr 相机标定（pinhole-equi版)）

Bash # 【每条新 BAG 只修改这一行】 BAG_NAME="camera_cp00016_0922_1545.bag" # 自动生成对应机器目录：例如 cp-00016/camera CP_ID="$(printf '%s\n' "$BAG_NAME" ¦ sed -E 's/^camera_cp([0-9]+)_.*/cp-\1/')" RESULT_DIR="/home/key/文档/kalibr/data/$CP_ID/camera" SOURCE_BAG="/home/key/文档/kalibr/data/live_bags/$BAG_NAME" # 首次标定：把已校验的 BAG 放入对应机器目录。 # 同一 BAG 再次运行时，发现它已在目标目录，会直接复用。 mkdir -p "$RESULT_DIR" if [ -f "$SOURCE_BAG" ]; then mv -v "$SOURCE_BAG" "$RESULT_DIR/$BAG_NAME" elif [ -f "$RESULT_DIR/$BAG_NAME" ]; then echo "复用已有 BAG：$RESULT_DIR/$BAG_NAME" else echo "找不到 BAG：$SOURCE_BAG" exit 1 fi # 运行 Kalibr；自动焦距失败时，终端才会要求输入 556 后回车。 docker exec \ -e BAG_NAME="$BAG_NAME" \ -e BAG_DIR="/data/$CP_ID/camera" \ -it kalibr bash -lc ' set -e source /opt/ros/noetic/setup.bash source /catkin_ws/devel/setup.bash export KALIBR_MANUAL_FOCAL_LENGTH_INIT=1 export MPLBACKEND=Agg unset DISPLAY cd "$BAG_DIR" rosrun kalibr kalibr_calibrate_cameras \ --target /data/april_6x6.yaml \ --models pinhole-equi pinhole-equi pinhole-equi pinhole-equi \ --topics \ /fisheye/left/image_raw/compressed \ /fisheye/right/image_raw/compressed \ /fisheye/bright/image_raw/compressed \ /fisheye/bleft/image_raw/compressed \ --bag "$BAG_DIR/$BAG_NAME" \ --bag-freq 10.0 \ --dont-show-report '.

说明：四路 Topic 顺序、AprilGrid YAML、四个 pinhole-equi 与 --bag-freq 10.0 为固定项。

## 异常处理 仅在录制被意外中断时使用

操作窗口：Linux 工作站终端。

Plain Text ssh -tt orangepi@192.168.10.109 ' source /opt/ros/noetic/setup.bash rosbag reindex /media/orangepi/sdcard/data/kalibr_bags/camera_cp0004_0920_1640.bag.active rosbag info /media/orangepi/sdcard/data/kalibr_bags/camera_cp0004_0920_1640.bag.active '.

说明：只在文件仍为 .bag.active 时使用。正式标定建议重新完整录制。

# 三 附：头环三项标定流程

# 四 附：camera_imu   DS验证命令

Bash ( set -Eeuo pipefail trap 'rc=$?; trap - EXIT; printf "\n脚本结束，退出码：%s\n" "$rc"; read -r -p "按回车关闭窗口..." _ ¦¦ true; exit "$rc"' EXIT # ===== 每次使用时只修改以下 2 行 ===== DEVICE_DIR="cs-0001" # 设备目录，例如 cs-0001、cp-00016 CAMERA_BAG_BASE="camera_cs0001_1010_1600" # 第一步 BAG 名，不带 .bag；用于找到对应 camchain # ====================================== DATA_ROOT="/home/key/文档/kalibr/data" CAMERA_IMU_DIR="$DATA_ROOT/$DEVICE_DIR/camera_imu" # 第一步相机标定结果、第二步 IMU 配置、标定板 CAMCHAIN="$DATA_ROOT/$DEVICE_DIR/camera/${CAMERA_BAG_BASE}-camchain.yaml" IMU_YAML="$DATA_ROOT/$DEVICE_DIR/imu/imu.yaml" TARGET="$DATA_ROOT/april_6x6.yaml" # 自动识别第三步动态 BAG；必须恰好有一个 camera_imu*.bag mapfile -t CAMERA_IMU_BAGS < <( find "$CAMERA_IMU_DIR" -maxdepth 1 -type f \ -name 'camera_imu*.bag' ! -name '*.orig.bag' -print ¦ sort ) if [[ "${#CAMERA_IMU_BAGS[@]}" -eq 0 ]]; then echo "没有找到第三步 Camera-IMU BAG，当前不能运行联合标定。" >&2 echo "请先录制 BAG 并放入：$CAMERA_IMU_DIR" >&2 exit 1 elif [[ "${#CAMERA_IMU_BAGS[@]}" -gt 1 ]]; then echo "找到多个 Camera-IMU BAG，请先确认本次要使用的文件：" >&2 printf ' %s\n' "${CAMERA_IMU_BAGS[@]}" >&2 exit 2 fi SOURCE_BAG="${CAMERA_IMU_BAGS[0]}" BAG_NAME="$(basename "$SOURCE_BAG")" BAG_BASE="${BAG_NAME%.bag}" # 所有输出进入独立测试目录，不覆盖第一步、第二步或已有第三步结果 RUN_TAG="ds_camera_imu_5hz_$(date -u +%Y%m%dT%H%M%SZ)" RUN_DIR="$CAMERA_IMU_DIR/ds_joint_tests/$RUN_TAG" RUN_DIR_INNER="/data/$DEVICE_DIR/camera_imu/ds_joint_tests/$RUN_TAG" CAMCHAIN_INNER="/data/$DEVICE_DIR/camera/${CAMERA_BAG_BASE}-camchain.yaml" IMU_INNER="/data/$DEVICE_DIR/imu/imu.yaml" TARGET_INNER="/data/april_6x6.yaml" BAG_INNER="$RUN_DIR_INNER/$BAG_NAME" CAMCHAIN_OUT="$RUN_DIR/${BAG_BASE}-camchain-imucam.yaml" IMU_OUT="$RUN_DIR/${BAG_BASE}-imu.yaml" RESULTS_OUT="$RUN_DIR/${BAG_BASE}-results-imucam.txt" REPORT_OUT="$RUN_DIR/${BAG_BASE}-report-imucam.pdf" LOG_FILE="$RUN_DIR/run.log" BAG_INFO="$RUN_DIR/bag_info.txt" METADATA="$RUN_DIR/run_metadata.txt" TOTAL_START_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" TOTAL_START_SECONDS="$(date +%s)" for FILE in "$CAMCHAIN" "$IMU_YAML" "$TARGET" "$SOURCE_BAG"; do if [[ ! -s "$FILE" ]]; then echo "文件不存在或为空，停止：$FILE" >&2 exit 1 fi done DS_CAM_COUNT="$(grep -Ec '^[[:space:]]*camera_model:[[:space:]]*ds[[:space:]]*$' "$CAMCHAIN" ¦¦ true)" if [[ "$DS_CAM_COUNT" -ne 4 ]]; then echo "相机链中 DS 相机数量为 $DS_CAM_COUNT（预期 4），停止。" >&2 echo "相机链：$CAMCHAIN" >&2 exit 1 fi if [[ "$(docker inspect -f '{{.State.Running}}' kalibr 2>/dev/null)" != "true" ]]; then echo "Kalibr 容器 kalibr 未运行，请先启动容器。" >&2 exit 1 fi if [[ -e "$RUN_DIR" ]]; then echo "测试目录已存在，为避免覆盖而停止：$RUN_DIR" >&2 exit 2 fi mkdir -p "$RUN_DIR" cp -v -- "$SOURCE_BAG" "$RUN_DIR/$BAG_NAME" SOURCE_SHA256="$(sha256sum "$SOURCE_BAG" ¦ awk '{print $1}')" COPY_SHA256="$(sha256sum "$RUN_DIR/$BAG_NAME" ¦ awk '{print $1}')" if [[ "$SOURCE_SHA256" != "$COPY_SHA256" ]]; then echo "BAG 副本 SHA256 校验失败，停止。" >&2 exit 3 fi # 先确认副本可读且包含四路压缩图像与 IMU topic set +e docker exec \ -e BAG_INNER="$BAG_INNER" \ kalibr bash -lc ' source /opt/ros/noetic/setup.bash rosbag info "$BAG_INNER" ' 2>&1 ¦ tee "$BAG_INFO" BAG_INFO_EXIT_CODE="${PIPESTATUS[0]}" set -e if [[ "$BAG_INFO_EXIT_CODE" -ne 0 ]]; then echo "rosbag info 无法读取 BAG；尚未启动联合标定。" >&2 exit 4 fi for TOPIC in \ /fisheye/left/image_raw/compressed \ /fisheye/right/image_raw/compressed \ /fisheye/bright/image_raw/compressed \ /fisheye/bleft/image_raw/compressed \ /imu_data_raw do if ! grep -Fq "$TOPIC" "$BAG_INFO"; then echo "BAG 缺少所需 topic：$TOPIC；尚未启动联合标定。" >&2 exit 5 fi done echo echo "第三步：DS Camera-IMU 联合标定" echo "设备目录：$DEVICE_DIR" echo "第一步相机链：$CAMCHAIN" echo "第二步 IMU 配置：$IMU_YAML" echo "第三步动态 BAG：$SOURCE_BAG" echo "抽帧频率：5.0 Hz（原文档参数）" echo "时间偏移：Kalibr 默认范围" echo "独立输出目录：$RUN_DIR" echo "运行期间请勿关闭终端或按 Ctrl+C。" KALIBR_START_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" KALIBR_START_SECONDS="$(date +%s)" set +e docker exec \ -e BAG_INNER="$BAG_INNER" \ -e CAMCHAIN_INNER="$CAMCHAIN_INNER" \ -e IMU_INNER="$IMU_INNER" \ -e TARGET_INNER="$TARGET_INNER" \ -e RUN_DIR_INNER="$RUN_DIR_INNER" \ kalibr bash -lc ' set -e source /opt/ros/noetic/setup.bash source /catkin_ws/devel/setup.bash export MPLBACKEND=Agg unset DISPLAY cd "$RUN_DIR_INNER" rosrun kalibr kalibr_calibrate_imu_camera \ --target "$TARGET_INNER" \ --imu "$IMU_INNER" \ --cams "$CAMCHAIN_INNER" \ --bag "$BAG_INNER" \ --bag-freq 5.0 ' 2>&1 ¦ tee "$LOG_FILE" KALIBR_EXIT_CODE="${PIPESTATUS[0]}" set -e KALIBR_END_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" KALIBR_SECONDS="$(( $(date +%s) - KALIBR_START_SECONDS ))" CAMCHAIN_OK=no IMU_OUT_OK=no RESULTS_OK=no REPORT_OK=no [[ -s "$CAMCHAIN_OUT" ]] && CAMCHAIN_OK=yes [[ -s "$IMU_OUT" ]] && IMU_OUT_OK=yes [[ -s "$RESULTS_OUT" ]] && RESULTS_OK=yes [[ -s "$REPORT_OUT" ]] && REPORT_OK=yes if [[ "$KALIBR_EXIT_CODE" -eq 0 && "$CAMCHAIN_OK" == yes && "$IMU_OUT_OK" == yes && "$RESULTS_OK" == yes && "$REPORT_OK" == yes ]]; then STATUS=SUCCESS_COMPLETE else STATUS=INCOMPLETE_OR_FAILED fi TOTAL_END_UTC="$(date -u +%Y-%m-%dT%H:%M:%SZ)" TOTAL_SECONDS="$(( $(date +%s) - TOTAL_START_SECONDS ))" { printf 'step=step3_camera_imu_joint_calibration\n' printf 'device_dir=%s\n' "$DEVICE_DIR" printf 'camera_bag_base=%s\n' "$CAMERA_BAG_BASE" printf 'camera_model_source=step1_camchain_DS_x4\n' printf 'camera_imu_bag=%s\n' "$SOURCE_BAG" printf 'bag_frequency_hz=5.0\n' printf 'timeoffset_padding=Kalibr_default\n' printf 'bag_sha256=%s\n' "$COPY_SHA256" printf 'total_start_utc=%s\n' "$TOTAL_START_UTC" printf 'total_end_utc=%s\n' "$TOTAL_END_UTC" printf 'total_wall_seconds=%s\n' "$TOTAL_SECONDS" printf 'total_wall_minutes_seconds=%s分%s秒\n' \ "$((TOTAL_SECONDS / 60))" "$((TOTAL_SECONDS % 60))" printf 'kalibr_start_utc=%s\n' "$KALIBR_START_UTC" printf 'kalibr_end_utc=%s\n' "$KALIBR_END_UTC" printf 'kalibr_wall_seconds=%s\n' "$KALIBR_SECONDS" printf 'kalibr_exit_code=%s\n' "$KALIBR_EXIT_CODE" printf 'camchain_imucam_exists=%s\n' "$CAMCHAIN_OK" printf 'imu_output_exists=%s\n' "$IMU_OUT_OK" printf 'results_imucam_exists=%s\n' "$RESULTS_OK" printf 'report_imucam_exists=%s\n' "$REPORT_OK" printf 'completion_status=%s\n' "$STATUS" } ¦ tee "$METADATA" echo echo "=== 本次测试文件 ===" ls -lh "$RUN_DIR" echo echo "状态：$STATUS" echo "结果目录：$RUN_DIR" [[ "$STATUS" == SUCCESS_COMPLETE ]] )